SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE: CSA0713	COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I LEVIN ANISH(192511223) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:LEVIN ANISH A

Annexure A

ANALYTICAL PROBLEM SOLVING

- Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

- a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

- A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

- a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a

/27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full

notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

- A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understandin g | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding ; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formul a with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |

ANSWERS:

1. CRC (Polynomial = X³ + 1)

Given:

Message = 11100011

Generator Polynomial = X³ + 1 = 1001

(i) Transmitted Message

Append 3 zeros:

11100011 → 11100011000

Divide by 1001 (modulo-2 division).

CRC Remainder = 010

Therefore,

Transmitted message = 11100011010

(ii) Leftmost bit is inverted

Received message:

01100011010

Receiver again divides by 1001.

The remainder is not equal to 000.

Answer:

CRC remainder ≠ 000

Receiver detects an error because the remainder is non-zero.

2(a) Throughput

Given:

Bandwidth = 10 Mbps

Frames/min = 12,000

Bits/frame = 10,000

Formula

Throughput = (Frames × Bits)/Time

= (12000 × 10000)/60

= 120,000,000/60

= 2,000,000 bps

= 2 Mbps

Answer:

Throughput = 2 Mbps

2(b) Channel Capacity

Given

Bandwidth = 3000 Hz

SNR = 3162

Shannon Formula

C = B log₂(1 + SNR)

= 3000 log₂(3163)

≈ 3000 × 11.63

≈ 34,890 bps

≈ 34.9 kbps

Answer:

Channel Capacity ≈ 34.9 kbps

3. Hamming (12,8) Code

Answer:

Hamming (12,8) adds 4 parity bits to 8 data bits.

It can detect and correct one-bit errors.

If one bit changes during transmission, the syndrome identifies the error position.

The receiver corrects the error automatically without retransmission.

4(a) IPv4 Subnetting

Given:

Network = 172.16.10.0/24

Subnet Mask = /27

Number of Subnets

Borrowed bits = 27 − 24 = 3

Subnets = 2³ = 8

Hosts per Subnet

Host bits = 32 − 27 = 5

Hosts = 2⁵ − 2

= 30 usable hosts

Subnet Range

Each subnet increases by 32.

Subnet

Range

1

0–31

2

32–63

3

64–95

4

96–127

5

128–159

6

160–191

7

192–223

8

224–255

IP = 172.16.10.78

Falls in:

64–95

Network Address:

172.16.10.64

Broadcast Address:

172.16.10.95

IP = 172.16.10.95

Yes.

It is the broadcast address of the same subnet.

Final Answer

Total Subnets = 8

Usable Hosts = 30

Network Address = 172.16.10.64

Broadcast Address = 172.16.10.95

172.16.10.95 belongs to the same subnet (broadcast address).

4(b) IPv6

Given

2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed Form

2001:db8::ff00:42:8329

Expanded Form

2001:0db8:0000:0000:0000:ff00:0042:8329

Network Prefix (/64)

2001:0db8:0000:0000::/64

Host Addresses

Host bits = 128 − 64 = 64

Total hosts

= 2⁶⁴

= 18,446,744,073,709,551,616

5. Networking Problem

Primary Problem

Flow Control

Explanation

The server sends data much faster than some student devices can receive.

Slow receivers cannot process packets quickly.

Receiver buffers become full (buffer overflow).

Packets are dropped.

Retransmissions increase network traffic.

Downloads become slow and delayed.

Solution

Use Flow Control (Sliding Window/TCP Flow Control) so the sender adjusts its transmission speed according to the receiver's capacity.

Final Answers (Short Form)

1(i) Transmitted message = 11100011010

1(ii) CRC remainder ≠ 000 → Error detected

2(a) Throughput = 2 Mbps

2(b) Channel Capacity ≈ 34.9 kbps

3) Hamming (12,8) detects and corrects one-bit errors using 4 parity bits.

4(a)

Subnets = 8

Hosts/Subnet = 30

Network = 172.16.10.64

Broadcast = 172.16.10.95

172.16.10.95 = Same subnet (broadcast address)

4(b)

Compressed = 2001:db8::ff00:42:8329

Expanded = 2001:0db8:0000:0000:0000:ff00:0042:8329

Prefix = 2001:db8:0:0::/64

Hosts = 2⁶⁴ = 18,446,744,073,709,551,616

5) Problem = Flow Control. Fast sender + slow receiver → buffer overflow, retransmissions, and delays.